# Tab 1

# Applications & Integration of Gravitational Batteries

Our project - The Feasibility and Integration of Gravitational Storage-Regeneration Systems - explores how we can implement the principle of gravitational energy storage in pre-existing infrastructure; thus, eliminating the main restriction behind current solutions of Gravitational Batteries.

The main focus of our project was to see whether the systems that we invented is able to perform closer or as well as the current gravitational energy storage such as pumped hydro and gravitational machine build by companies like Energy Vault

This document focuses on how we were able to develop and design our four systems inspired from the preexisting infrastructures and laws around us. Our main inspiration for our project was elevators, the natural buoyancy of water and electromagnetic propulsion & levitation

By Kavin Paturu Muralirkishnan, Aiman Ullah & Amaan Khan

Dual Weight Regeneration System -  #1

Main Idea:

- When we came across the area of block gravitational energy storage made widely by companies like Energy Vault and Graviticity, we wondered why wasn’t the storage widely used or available.

- After further research and interviews with engineers working in the companies, we found out that gravity storage was location specific and had huge infrastructure costs, which significantly hindered the development of the concept.

- We thought why can’t we develop gravity based systems so that it can be implemented in preexisting infrastructure, that’s when we came across elevators in commercial buildings.

- Elevators are built in heavy blocks that move up and down in almost every commercial building, which inspired us to develop a system to harness the kinetic energy of the moving elevator.

Background on infrastructure of elevators:

- The cab is the block or space where people get in to travel up and down a singular motor is used to bring the cab up and down.

- Due to the high load on one side of motor a counterweight is used to balance out the load force

- Counterweight is usually ½ of the weight of the cab at maximum capacity

- Both use a guide rail/ guide roller system to control the falling motion of the cab and also to stabilize both the masses.

- There are two types of elevators based on their control system: MRL elevator and machine room elevator

- MRL elevators does not have a machine room to control the elevator falling motion

- Machine room elevator has a separate room on top of the hoistway to control the motion of elevator

- We also looked into the structure of pulley used in elevator and also the type of motor used to lift the elevator up and down

- There is a space beneath the elevator shaft for spring breaking and space above the elevator shaft so that it has some clearance, especially for MRL elevator

Our solution and integration into elevators:

- Our idea is to convert the kinetic energy of both cab and counterweight into electricity, when the elevator is in use (mainly during daytime)

- We will achieve this by retrofitting a motor/generator onto the main sheave pulley that is directly connected to both cab and the counterweight.

- Due to our project's focus on energy storage, we thought why can’t we store potential energy when the elevator is not in use, and when there is electricity needed for the building we would be able to drop it at a control rate.

- Even though we are dropping it at a control rate when the cab goes down the counterweight goes up meaning as one mass is generating electricity, the other mass is storing potential energy, which can also happen vice versa.

- Due to two weights connected to each other you will be needing less energy to pull the weight back up, but you are still storing the same amount of potential energy.

Our solution and integration into vertical lift bridges

- Vertical lift bridges also have vertically moving counterweights which are mainly used to lift the bridge up so ships can pass through.

- Generally vertical liftbridges have 2 counterweights to lift the bridge up meaning, when energy is needed we could drop both the counterweights down to generate energy, similar to an elevator

- When there is a deficit of energy in the grid counterweight can be dropped to lift the bridge, and when there is excess amount of energy the bridge can be dropped lifting the counterweight.

- These systems are part of a unique variety of storage systems called the Lift energy system and gravity energy system.

Variable Counterweight - #2

Main Idea:

- In a system with a weight and counterweight, the counterweight’s purpose is to reduce the work needed to lift the other weight.

- These kinds of systems work best when both weights are equal, allowing minimal energy to be used when tipping the balance.

Integration into Elevators:

- In an elevator, the counterweight’s purpose is to reduce the load on the elevator motor. The motor would function at its best when both the cab and the counterweight is of equal weight, allowing the motor to use minimal energy to tip the balance.

- A system where the counterweight can be adjusted based on the weight of the cab would allow the elevator to run with minimal energy due to the reduced workload on the motor.

- The variable counterweight system changes the mass of the counterweight depending on the change in mass of the cab by dropping modular weights directly to the counterweight via cables, making the effective mass of the system as close to zero.

- The motion of the modular weight is converted into electricity to regenerate energy through two motors located at the top of the hoistway and small rollers that power generators inside the modular counterweight itself.

- Current patented systems involve moving sand and water to vary counterweights, but these systems would be difficult to implement in elevators due to the complexity and safety risks.

Buoyant Gravity Battery - #3

Main Idea:

- The main concept behind this battery is to use the natural buoyancy of water to lift the weight back up

- During our research we encountered a buoyant battery where it stores energy by pulling a buoyant weight to the sea floor, and when there is energy needed, the buoyant mass is released so that the net force will accelerate the mass upwards

- The buoyant battery is mainly used in pair with tidal power and also used in coastal areas

- Problem: the acceleration of the net force is extremely low compared to gravitational acceleration, so the buoyant energy storage is not as efficient, in terms of discharge efficiency, as gravitational energy storage

Integration into water tanks/water towers:

- Rural villages use huge elevated, silo like water tanks to distribute water throughout the village just by using potential energy of water.

- Due to this unique structure of the water tanks, our buoyant gravity battery can easily fit into this specific infrastructure, enabling our technology to be more accessible to rural communities

- When the water is released for distribution, the mass will drop into the created empty space in the water tanks generating energy

- When there is an excess surplus of energy, generally, the ground water is pumped all the way up the tank to store water, meaning at the same time the buoyant mass will also charge.

Electromagnetic Gravity Battery - #4

Main Idea:

- Inspired by the levitation and propulsion systems of maglev trains, this system utilizes tubular linear motors to move heavy masses. Its primary goal is to break the current efficiency limitations of gravitational storage (80–85%) to reach the levels of lithium-ion batteries (90–95%) [8].

Integrations into other Gravitational designs

This system can be implemented into other existing gravitational designs, including each of the three above, to increase their efficiency.

Electromagnetic Gravity Battery - #4

Main Idea:

- Inspired by the levitation and propulsion systems of maglev trains, this system utilizes tubular linear motors to move heavy masses. Its primary goal is to break the current efficiency limitations of gravitational storage (80–85%) to reach the levels of lithium-ion batteries (90–95%).

Integration into other gravitational batteries:

- The main purpose of this technology is to improve the general efficiency of gravitational batteries by introducing a more efficient motor - tubular motor.

- The tubular motor converts electrical energy directly into linear motion based on Lenz’s law/right hand rule

- The tubular motor system is generally better than the traditional motor hoist drum system used in block/lift gravitational systems (made and used by Energy Vault) due to decrease in frictional points

Future Aspect for electromagnetic system:

- The main idea and reasoning behind the electromagnetic system was to introduce a new potential energy (electromagnetic potential energy) so that we would be able to increase the roundtrip efficiency of gravitational batteries.

- By introducing the electromagnetic potential the object will be able to accelerate at greater rate than free fall objects

- The electromagnetic acceleration can be seen through rail technologies which accelerates small metal object using electromagnetic coils

- Same can be seen in Maglev trains, where the train uses special linear motors to reach high velocities in seconds

- We coined the future system as hyperloop electromagnetic gravitational energy storage

# Possible Elevator Integration

# Possible Integration in Elevators

Regenerative elevators were a thing

- Otis and KONE are companies that make regen systems/regen drives

- Otis introduced this in 2005 as part of its Gen2 elevator system

  - Regen Drive Final Facts Sheet

    - Energy savings of up to 75%

    - Low Harmonic Distortion - Below 5%

Regenerative systems

- Elevators have huge potential energy when about to descend

  - Typical elevator at 1 floor: PE = 29,430 J

    - Height: 3m

    - Mass: 1000kg

    - Gravity: 9.81 m/s

  - Descending 1 floor: KE = 1125 J

    - Mass = 1000 kg

    - Velocity = 1.5 m/s

  - The kinetic energy is only 4% of the potential energy. The rest of the energy is converted to mechanical friction and dissipated as heat

- Current regenerative systems only focus on the elevator, EcoDrop can focus on both the elevator as well as the counterweight in order to achieve bidirectional regeneration

  - Needs to be able to detect direction of motion/force

  - Space constraints in elevator shaft

Green energy solutions to invent

- Elevator

  - “Purpose fall” of counterweight when the cab is empty and the elevator is not in use. Instead of a stand-still elevator, drop it and harness the potential energy

  - Use guide wheels and guide rail as a gear system and capture the rotational motion to then spin a generator.

  - Use solar panels to help bring the elevator back up and store it there

    - Problem is that it is probably used for the internal grid of the building

  - Add ventilation design to elevator shaft and add propellers to the cab so that when the elevator goes up it slightly propels as well as when the elevator descends, it generates electricity as it spins. Same system could be added to the counterweight

    - There could also be a drag brake support system so that we could use the air flow to slightly slow down the elevator as well.

  - Add a counterweight system that changes based on the load in the cab. This could be done by using sand or fluids pumped in and drained out to the counterweight

    - Add a filtration system in the counterweight that filters “bad water” to then drinkable/usable water. → Can take advantage of the rainwater collection system in the Virginia Tech Building.

    - Or, have a detachable weight transport system. There is a box at the top of the hoistway that drops heavy barbell weights using a pulley system and center beam. Each barbell weight has a hole where the beam passes through. Inside each weight are rollers centered at the hole where they roll along the beam when descending. The weight has an inner electronic system where as the rollers rotate, they generate electricity with the mini generators attached to each roller. There are supercapacitors within to store the electricity.

  - Apply a Hybrid Energy Storage System(HESS) - Use Ultracapacitor Energy Storage (UCES) and Battery Energy Storage(BES)

    - Need to implement green battery methods

      - Recycling batteries to reclaim the metals in the batteries

      - Ethical sourcing and suppliers

      - Different cathode and anode materials to reduce mining

    - Spikes in breaking can go into ultracapacitor and slowly go into battery to slow degradation process (Battery Management System)

- Sound energy to electricity

  - Use sound vibrations to a oscillate magnet through a coil to induce a current and generate electricity

    - Achieved through resonance and noise pollution

      - Invention could be placed in an urban area where there is a lot of noise, or a factory, or traffic

  - Energy from noise oscillating magnetic fields is theoretically possible, especially via resonant quantum spin transitions or through classical mechanical effects on magnetic moments. However, the energy scales involved are general small at ambient noise levels. Efficient conversion depends on matching resonance conditions and material properties.

References

- Elevator system including regenerative drive and rescue operation circuit for normal and power failure conditions  (Google Patent)

- Elevator Regenerative Energy Applications with Ultracapacitor and Battery Energy Storage Systems in Complex Buildings (Study)

  - cited

- Thermoelectric thermal management system for the energy storage system in a regenerative elevator (Google Patent)

- Associates of Computing Machinery

- CSCW 2026 Publishing site - Paper ready for publication

- Washington Academy of Sciences

- Virginia Junior Academy of Sciences

- Supercapacitor-Based Energy Storage in Elevators to Improve Energy Efficiency of Buildings (Study)

  - cited

- An Evaluation of Energy Storage Cost and Performance Characteristics (Study)

  - cited

- Varying Elevator Counterweight (Patent)

- https://schindlerplan.com/product-category/model/?cat=3

Questions and research topics

- What type of building are we implementing the gravity battery energy storage system? - calculate kwh usage on elevator system on each type of building and choose the best one

- What would be the average specs of our elevator such as speed, height, load: refer to laws on how to design the elevator

- How to constantly change the weight of the counterweight so that maximum efficiency is achieved? -  Ideas: patented ideas such as using a fluid based system in order to change the counterweight’s weight

Conclusions & Ideas

- Not going to use batteries in energy storage to use in peak hours, ruins our entire purpose of project

- Ultra capacitors stored sudden energy burst as electrical does not convert to chemical - best storage capacity for now

- Hydrogen fuel cell technology: stores energy in form of hydrogen capsules - proficient in hydraulic elevators due to its usage of water

- Compressed air storage system  - can be used in counter weight in order for constant change of weight balance so the elevator will constantly work at its best  - works for the large scale

- Flywheels storage system

Further implementation on the elevator system

- Adding propellers perpendicular to the cab and the counterweight in order to keep the momentum of the falling and lifting of the cab and counter weight, acts as wind power inside the elevator system also producing energy

Elevator design Specifications

- https://schindlerplan.com/product-category/model/?cat=3 - elevator design specifications

- Design specs

  - High rise/ Mid rise buildings - 7 or more stories

  - Capacity of the cab -   4000 lbs or 3500 lbs

  - Empty Cab weight - 1800

  - Weight of counterweight:

  - Cab height - 9-10 feet

  - Cab width - 7-8 feet

  - Cab depth: 5-6 feet

  - Cab speed: 750 fpm - 1000 fpm

  - Model height: 150 feet - 10 to 13 stories

  - Need a machine room - because need due to high to mid rise: MRL vs Machine room

  - Type of buildings - apartment buildings and office buildings for frequent user

  - Traction elevators

  - Counterweight - rear would do: High speed elevators for high rise buildings

  - Number of stories - 10 to 13

- The model hoist away drawing - The hoistway dimensions

- Engineering design 2

  - In seismic zone - no

  - Unit quantity - 1

  - Service type - general purpose

  - Cab capacity - 4000 lbs

  - Speed - 500 fpm

  - Door type - single speed center opening

  - Cab height - 10 ft

  - Rear openings - No

  - Travel limit - 100 ft

  - Rear counterweight

  - Results

    - Hoistway Width - 9’ 4”

    - Hoistway Depth = 7’ 8’’ or 92 inches

    - Pit Depth - 5’ 5’’ or 65 inches

    - Overhead - 17’ 5’’ - 209 inches

- Representative structure with 7 or 8 stories dimensions

  - Total Representative height - 100 ft or 1200 in

  - Overhead (includes final floor to hoistway bottom) - 17 ft or 204 in

  - Machine room height- 7 ft or 84 inches

  - Pit depth - 5’5’’ or 65 in

  - Travel limit - 70.58 ft or 847 inches

  - Per story height goal is 10 feet

    - If 8 stories - per story height will be 8.82 ft or 105.875 inches ~ 9 ft

    - If 7 stories - per story height will be 10.08 ft or 121 inches ~ 10 ft

- Representative structure with 100 feet travel limit - 10 ft per story, 10 stories

  - Overhead (includes final floor to hoistway bottom) - 17 ft or 204 in

  - Machine room height- 7 ft or 84 inches

  - Pit depth - 5’5’’ or 65 in

  - Total Representative height - 129.42 ft or 1553 in

  - Actual elevator in VT - 4000 lbs Cab inner dimensions

    - Inside cab height: 9ft

    - Width: 92 in or 7” 8’

    - Depth: 65 in

  - 4000 lbs Schindler

    - Inside cab height: 8 - 10 ft

    - Cab width: 8” 0 ⅛’ ft

    - Cab depth: 5” 4’ ft

  - 4000 lbs Gen 3 peak

    - Cab depth: 5” 5’ ft

    - Cab Height: 8 ft - 9” 7’ ft

    - Cab Width: 7” 8” ft

  - Actual cab dimensions for representative model

    - INTERIOR DIMENSIONS:

    - Cab Depth: 5’ 5’’  or 65 inches

    - Cab width: 7’ 8”

    - Cab Height: 9” 2’ or 110 inches

    - ENTIRE CAB DIMENSIONS:

    - Cab wall thickness: 3.5 in

    - Final cab depth: 65 inches or 5’ 5”

    - Final cab width: 7’ 8”

    - Final cab Height: 9.75 ft or 117 inches

    - Clearance space in the hoistway (depth): 2’ 3”

    - Clearance space in the hoistway (width): 1’ 3”

- FINAL Representative elevator dimensions

  - Cab

    - Final cab depth: 65 inches or 5’ 5”

    - Final cab width: 7’ 8”

    - Final cab Height: 9.75 ft or 117 inches

    - Clearance space in the hoistway (depth): 2’ 3”

    - Clearance space in the hoistway (width): 1’ 8”

  - Hoistway

    - Total Representative height - 100 ft or 1200 in

    - Overhead (includes final floor to hoistway bottom) - 17 ft or 204 in

    - Machine room height- 7 ft or 84 inches

    - Pit depth - 5’5’’ or 65 in

    - Travel limit - 70.58 ft or 847 inches

    - Per story height goal is 10 feet

      - If 8 stories - per story height will be 8.82 ft or 105.875 inches ~ 9 ft

    - Hoistway Width - 9’ 4”

    - Hoistway Depth = 7’ 8’’ or 92 inches

  - Other

    - In seismic zone - no

    - Unit quantity - 1

    - Service type - general purpose

    - Cab capacity - 4000 lbs

    - Speed - 500 fpm

    - Door type - single speed center opening

    - Cab height - 10 ft

    - Rear openings - No

    - Rear Counterweight

- FINAL Model Dimensions

  - Total Height - 6 ⅔ feet or 80 inches

  - Hoistway Width is 18 ⅔ inches

  - Hoistway Depth is 15 ⅓ inches

  - Cab

    - Depth: 10.8 inches

    - Final cab width:15 ⅓ inches

    - Final cab Height: 19.5 inches

  - Clearance Depth: 4.533 inches

  - Rear Counterweight

# Tab 3

### **Applications of Dual Weight Systems for Gravitational Energy Storage and Regeneration**

This document outlines three innovative gravitational battery systems designed to store and regenerate energy by leveraging natural forces, electromagnetic propulsion, and adaptive mechanical loads.

---

#### **1. Buoyant Gravitational Energy Storage System**

This system is inspired by the law of buoyancy as a natural force to return weights to a high-potential state with minimal energy input [1]. While initial research indicated it may be the "third most efficient" among studied systems, its integration potential into existing infrastructure is significant [1, 2].

*   **Mechanism:**

The system utilizes buoyant masses that float. During periods of excess energy, these masses are pulled down to a seabed or pushed up by filling a chamber with water [1, 2]. To generate energy, the chamber is discharged, and the weight is dropped; this is effective because gravitational acceleration is greater than buoyant acceleration [2]. The system also utilizes the falling motion of water to generate additional electricity [2].

*   **Applications:**

*   **Coastal Regions:** Systems can be implemented in coastal areas where tides rise and fall. Buoyant masses can be released to accelerate upward, harnessing the natural movement of the sea [1, 3].

*   **Rural Community Water Tanks:** Many rural villages use high-altitude water tanks to distribute water via potential energy [4]. This system can be integrated into these tanks. During energy surpluses (e.g., when pumping groundwater), the buoyant mass is moved up, and during energy deficits, it is dropped to generate power [4, 5].

*   **Urban Water Pipes:** Vertical water pipes in apartment buildings can be fitted with small modular buoyant systems [5]. A redirected path with a motor uses excess energy to push a weight up into a special chamber; this weight is later dropped to produce electricity when needed [6].

---

#### **2. Electromagnetic Gravitational Battery**

Inspired by the levitation and propulsion systems of maglev trains, this system utilizes tubular linear motors to move heavy masses [7]. Its primary goal is to break the current efficiency limitations of gravitational storage (80–85%) to reach the levels of lithium-ion batteries (90–95%) [8].

*   **Mechanism:**

Standard gravitational systems are limited by mechanical losses and fixed gravitational acceleration [8, 9]. This system introduces a secondary downward acceleration through electromagnetic propulsion [9]. By using Faraday's Law and magnetic attractive forces, the mass is pulled downward faster than gravity alone would allow, which significantly increases the electricity produced during discharge [9, 10].

*   **Applications:**

*   **Universal Energy Storage:** Unlike systems dependent on water or specific topography, the electromagnetic system is not location-specific [10, 11].

*   **High-Efficiency Retrofitting:** This technology can be implemented into other existing gravitational designs to increase their efficiency and redefine how gravitational energy is stored and harvested [10, 11].

---

#### **3. Variable Counterweight Regeneration System**

This system is an evolution of the traditional elevator counterweight, which is typically fixed at 40–50% of maximum capacity [12]. The variable system replaces fixed weights with a dynamic "barbell" system [13].

*   **Mechanism:**

Using cameras or predictive algorithms, the system identifies the current load (e.g., the number of people in an elevator) [12]. It then adjusts the counterweight by dropping or adding small "barbell masses" to ensure the counterweight always equals the mass of the cab [12, 13]. The falling motion of these barbell masses is converted into kinetic energy via motors installed on the hoist [13].

*   **Applications:**

*   **Elevator Systems:** This is highly specific to elevators in high-rise buildings. By matching the counterweight to the cab load in real-time, the system generates electricity from the weight adjustments and reduces the energy required to move the cab [13, 14].

*   **Industrial Machinery:** The principle can be applied to other variable counterweight mechanisms, such as cranes, where loads are constantly changing and operational efficiency is paramount [15].

*   **Building Integration:** It serves as a singular system that uses algorithms to find the best route to drop weights and generate electricity throughout a building's vertical transit [14].