# Tab 1

FORMULAS

- E_grav = m * g * h

- P_mech = m * g * v    (instantaneous power)

- P_mech = (m * g * h) / t  (overall power)

- v = omega_drum * r

- omega = 2 * pi * (rpm / 60)

- rpm = (60 / (2 * pi)) * omega

- tau_drum = T * r

- T ≈ m * g

- tau_drum ≈ m * g * r

- P = tau * omega

- N = omega_gen / omega_drum

- omega_drum = omega_gen / N

- tau_gen = tau_drum / N

- tau_drum = N * tau_gen

- P_out = eta_gb * P_in

- P_mech_shaft = tau_gen * omega_gen

- E = Ke * omega_gen

- Ke = V_no_load / omega_rated

- V_term = E - I_load * Rint

- tau = Kt * I_load

- P_elec = V_term * I_load

- eta = P_elec / P_mech_shaft

- E_usable = eta_total * m * g * h

- sum_tau = J * alpha # alternative form of sum(tau) = I * alpha

- alpha = (delta_omega) / (delta_t)

- J_solid_cyl = 0.5 * M * R^2

- J_hoop = M * R^2

- J_total = sum(J_i)

- E_rot = 0.5 * J * omega^2

- t = h / v

- v = h / t

- omega_gen ≈ (V_target + I_load * Rint) / Ke

VARIABLES (symbol : meaning — typical units)
m : mass — kilograms (kg)
g : acceleration due to gravity — 9.81 m/s^2
h : drop height — meters (m)
v : linear speed at drum rim — meters per second (m/s)
t : drop time — seconds (s)
r : drum radius (m)
T : rope tension (N)
tau, tau_drum, tau_gen : torque (Nm). tau_drum is torque at the drum, tau_gen at the  generator shaft.
omega, omega_drum, omega_gen : angular speed (rad/s)
rpm : revolutions per minute
N : gearbox speed ratio = omega_gen / omega_drum (dimensionless)
J, J_total, J_i : moment of inertia (kgm^2). J is used here instead of I to avoid conflict with electric current.
alpha : angular acceleration (rad/s^2)
pi : 3.14159265... (dimensionless)
P, P_mech, P_mech_shaft, P_elec, P_in, P_out : power (watts, W)
E_grav, E_rot, E_usable : energy (joules, J)
E : open-circuit back EMF (volts, V)
Ke : back-emf constant (V per rad/s)
Kt : torque constant (Nm per A)
V_no_load : generator open-circuit voltage at a given rpm (V)
V_term, V_target : terminal voltage (V)
Rint : internal resistance of generator (ohm, Ω)
I_load : electrical load current (amps, A)
eta_gb : gearbox efficiency (fraction, 0..1)
eta_total, eta : overall system efficiency (fraction, 0..1)
sum_tau : sum of torques acting on the rotating system (Nm)
delta_omega : change in angular speed (rad/s)
delta_t : change in time (s)
M, R : mass and radius used in inertia formulas (kg, m)

# Revised Math Formulas

- Gravitational Potential Energy

  - E = mgh

- Average Mechanical Power Over the Drop

  - E = mgh/t

- Angular Velocity (ω) to RPM

  - RPM = ω * (60/2pi)

- RPM to Angular Velocity (ω)

  - ω = 2pi * (RPM / 60)

- Rope Tension

  - T = mg - ma - F_drag - F_mag

    - Tension = mg - net force - drag force - magnetic force

    - Derived from ma = mg - T - F_drag - F_mag

    - If steady slow descent → T ~ mg

- Torque

  - τ = rFsin(theta)

  - τ = I * α

    - Torque is the inertia of the object times its angular acceleration

- Drum Torque (τ) from Rope Tension (T)

  - τ = T * r

    - Drum torque = Rope tension * Drum radius

- Power from torque and angular speed

  - P = τ * ω

  - Use for drum, gearbox input, generator shaft

- Total Mechanical Resistance in Motor (AKA Winding Resistance)

  - R_winding = V_designated / τ_stall

    - Total resistance in the motor is found by dividing the voltage that the motor is designed to handle (nominal voltage) by the stall torque

- Resisting torque in the motor (Load torque)

  - τ_resisting = P_input / ω_motor

    - The resisting torque in the motor is the power fed into it divided by the angular speed of the motor

- Motor Torque constant (Unit - Nm/A)

  - Kt = (60/2pi) / Kv  ~ 9.5493/Kv

    - The motor torque constant is (60/2pi) divided by the motor speed constant

- Motor Back-EMF Constant (Unit - V * s / rad →Volt-seconds per radian)

  - Ke ~ Kt If the motor is a permanent magnet DC or BLDC

    - Motor Back-EMF constant is equal to the motor torque constant if the motor is PMDC or BLDC

- Torque of motor

  - τ_motor = Kt * I

    - Torque constant  multiplied by the current gives the torque of the motor

- Driving current (Starting current)

  - I_driving = (V_designated - RPM/Kv) / R_winding

    - The driving torque is the net voltage (nominal voltage - back emf voltage [RPM/Kv]) divided by the resistance of the motor (winding resistance)

- Driving Torque of motor

  - τ_driving = Kt * I_driving

  - Driving Torque MUST BE LESS THAN Stall torque

- Voltage of Motor

  - V_motor = Ke * ω_motor

    - Back-EMF constant multiplied by the angular speed of the motor gives the voltage outputted by the motor

- Maximum Power output of the Motor (Occurs around 50% of stall torque)

  - (V^2) / (4*R_winding)

    - The maximum power output of the motor is the square of the supply voltage divided by 4 times the winding resistance

- Necessary RPM of motor to generate a certain voltage

  - RPM_motor = V_required * Kv

    - Target voltage multiplied by the motor constant →Kv in RPM/V

    - Kv should be on the datasheet

    - You can also figure out Kv; for example, if the motor spec is 4000 RPM at 6V, then the Kv is 4000/6 or 666.67 RPM/V

- Gearbox Speed Multiplier

  - RPM of Motor / RPM of Drum

    - (Kv * V_goal)/

- Moment of Inertia for a SOLID cylinder

  - I = (½)(M*R^2)

- Moment of Inertia for a HOLLOW cylinder

  - I = (½)(M)(R_1^2 + R_2^2)

    - R_1 is the inner radius, R_2 is the outer radius

- Buoyant Force

  - Buoyant Force = weight of displaced fluid

    - The buoyant force of an object is the weight of the fluid it displaces once it submerges in the fluid. Basically mg of the displaced fluid.

    - F_B = D_fluid * V_object * g

- Hydraulic Power

  - P_hydraulic = D_fluid * g * Q * h

    - Hydraulic power required to lift a certain amount of fluid up to a height is the product of the density of the fluid, gravitational acceleration, the flow rate, and the height at which the fluid needs to be transported